# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Supply for Improving the Crime Scene Investigation Capacity of Turkey-Relaunch	p 1 /…

- Publication reference: Europe Aid/136649/IH/SUP/TR

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

ANNEX II:  TECHNICAL SPECIFICATIONS

1.1 GENERAL REQUIREMENTS

1.1.1	All the equipment shall be provided complete with the necessary accessories and/or parts to ensure that the unit is capable of operating to the required technical and quality specifications immediately. All specifications details listed within lot for each item is the minimum requirements. Any improvements on the specifications or additional features offered shall be clearly identified in the Tenderer’s offer.

1.1.2 	Any specification referring to or suggesting a particular product or manufacturer, in particular to types, models and brand names are always to be understood as “or equivalent”. Where equivalency shall be subject to technical evaluation, the respective documentation of equivalency, and - if appropriate - an assessment by an independent party shall be provided with the offer.

1.1.3	Equipment which allows upgrading of capacities (except the power capacity) shall be provided in such a way that upgrades can be performed by installing additional capacity without discarding the already installed capacities.

1.1.4 	Adequate documentation to assess the respective capacity, quality, compatibility and inter-connectivity of the product offered must be included with the offer.

1.1.5 	The Tenderer shall clearly state in the attached detailed Technical Specifications the proposed manufacturers and country of origin for each item and sub-item.

1.2	GENERAL TRAINING REQUIREMENTS

The Contractor shall prepare a detailed Training Programme including the scope of training, training schedule and instructors’ manpower assigned; the Training Programme and the scope of training materials shall be approved by the Beneficiary. The nominated trainees may provide their suggestions to the proposed Training Programme.

The Training Programme shall cover description of the equipment including installation, surveillance, inspection, operation, maintenance, periodical testing, instrument calibration and troubleshooting procedures, as well as necessary background information.

Training shall be based on the appropriate training documentation for the equipment and systems. This documentation shall be clearly labelled as Training Documentation. Complete sets of training manuals in Turkish shall be provided by the Contractor. The language of the training will also be Turkish; if not, interpretation/translation will be provided and paid by the Contractor.

The Contractor will be responsible to provide all the necessary service, technical personnel, product, equipment, documentation for the training programs. All costs related to the personnel training, such as teaching materials, travel and accommodation for the Contractor’s personnel, documentation translation costs and other miscellaneous expenses shall be borne by the Contractor.

The Contractor shall provide an adequate training for a number of end users as specified in the below table in a way that they will be able to operate the equipment without any help.  The lot/item numbers which the training is needed, minimum number of end users to be trained and the minimum period of the training is given in detail below.

| Item Number | Description | Training Requirement |
| --- | --- | --- |
| 1 | Crime Scene Investigation Vehicle | At least 5 staff of the Beneficiary personnel Duration: Minimum 1 day ( at least 4 hours) |

Starting date and time-table of the training courses shall be determined by mutual agreement between the Beneficiary and the Contractor.

2. VISIBILITY

All supplies must comply with the generic Visibility policies in force within the scope of external aid contracts financed from the EU general budget; tenderers will thus be aware that certain visibility rules apply and that the guidelines concerned may be found in the EUROPEAID website, at:

https://ec.europa.eu/europeaid/communication-and-visibility-manual-eu-external-actions_en

http://avrupa.info.tr/eu-funding-in-turkey/visibility-guidelines.html

Unless otherwise specified, all equipment shall have a solidly fixed and durable label, size 75 mm x 35 mm, as appropriate for each equipment with the standard EU logo as below:

INSTRUMENT FOR PRE-ACCESSION ASSISTANCE (IPA) PROGRAMME 2007-2013

- Europe Aid/136649/IH/SUP/TR

Lot No:

Item No	:

Serial No:

This project is co-funded by the European Union and the Republic of Turkey

Bu proje Avrupa Birliği ve Türkiye Cumhuriyeti tarafından finanse edilmektedir.

3. 	TECHNICAL SPECIFICATIONS TABLE

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Crime Scene Investigation Vehicle |  |  |  |
|  | Short description: Crime Scene Investigation Vehicle will be used by Crime Scene Investigation Teams for transportation, crime scene investigations, collection of evidence, storage and other forensic purposes. It shall be long van type chassis and consist of the cargo area and the driver/passenger cabin. The exact specifications are described below: 1.1. General Requirements: 1.1.1. The vehicles must provide maximal safety for all passengers and be supplied with all necessary equipment in compliance with Law on safety of the traffic. 1.1.2. The original electrical utility networks of vehicles will not be used in superstructure modification of vehicles. 1.1.3. The products which have certificates meeting HFFR/LSOH criteria of fire-proof cable class shall be used for all the electrical installations 1.1.4. All modifications of the vehicles shall be in compliance with Highway Traffic Laws Vehicle Production, Modification and Mounting Regulations and Vehicle Standards of Turkey. 1.1.5. The supplier will evaluate the supplied current, the quality of the current and the fluctuations of the current and take the necessary precautions to avoid damages to the equipment. 1.1.6. Whole system and system components and equipment in lot shall be compatible each other. 1.1.7 There shall not be any usernames, passwords, services, etc. providing access to the proposed products out of the Beneficiary Institution’s knowledge. 1.2. Technical Requirements of Vehicles: 1.2.1. Motor system shall be controlled electronically and suitable for cold weather conditions. 1.2.2. Motor Type: Euro 5 (five) or Euro 6 (six). 1.2.3. No of Cylinders: 4 (four). 1.2.4. Engine Power: Minimum 120 (one hundred and twenty) PS. 1.2.5. Fuel Tank Capacity: Min 70 (seventy) litres for fuel. 1.2.6. Torque: Minimum 350 (three hundred and fifty) Nm. 1.2.7. Driving gear: 4 (four) x 4 (four). 1.2.8. The vehicle’s capacity of load: min. 10 (ten) m³, max.13 (thirteen) m³. 1.2.9. ABS+EBD+ESP. 1.2.10. Min 5 (five) speed manual or automatic transmission. 1.2.11. Hill Holder. 1.2.12. Battery: Minimum 12 (twelve) V / 70 (seventy) Ah. 1.2.13. Alternator: Minimum 12 (twelve) V / 150 (one hundred and fifty) A. 1.2.14. Steering: Steering: The vehicles shall be equipped with the power assisted and functional steering (hydraulic or pneumatic) on the left side. 1.2.15. Axles/Springs/Frame: The suspension system shall include front and rear shock absorption by parabolic or coil springs. 1.2.16. Air Conditioning: The vehicles shall be equipped with manual or automatic climate controlled AC with dust and pollen filter which is driver operated. 1.2.17. The following items shall be furnished and in place in all vehicles. 1.2.17.1. Multi-speed fresh air ventilator with heater and windshield defogger system. 1.2.17.2. Seat belts for all seating positions. 1.2.17.3. Full floor covering. 1.2.17.4. Dome light, door activated. 1.2.17.5. Right and left side exterior rear view mirrors shall be electrically adjustable and shall be freeze-resistant, heated and controlled from inside of the cab. 1.2.17.6. AM/FM radio, AUX MP3 input. 1.2.17.7. Standard factory front and rear bumper with grille guard. Stainless front, rear and side protection barriers shall be mounted on the vehicle. 1.2.17.8. Speedometer and all necessary gauges. 1.2.17.9. Driver seat shall be adjustable with high back head support and arm rest. 1.2.17.10. Passenger seat shall be the manufacturer’s standard seat. 1.2.17.11. Passenger side glove compartment or storage net. 1.2.17.12. Twelve (12) V DC (Direct Current) power outlet in front compartment. 1.2.17.13. Electrical window lifters. 1.2.17.14. Alarm system, power locks with remote control and minimum 1 (one) air bag. 1.2.17.15. Rear-wheel drive with back up rear view camera (Back eye camera) and audial and visual warning parking sensor system. 1.2.17.16. Immobilizer. 1.2.17.17. Driver’s Manuel with Turkish option. 1.2.17.18. Integrated or external GPS navigation device with minimum 5 (five) "(inch) TFT LCD monitor/touchscreen included map of the Turkey. 1.2.17.19. Front and rear fog lamps, emergency braking support. 1.2.17.20. The vehicle shall have equipment needed to tow or to be towed. 1.2.17.21. Rear doors shall be able to open minimum 180 (one hundred and eighty) º (degrees) to make it easier to load and unload. 1.2.17.22. Motion sensor car camera/dash cam with SD card up to minimum 32 (thirty two) GB. 1.2.17.23. Standard Accessories for Each Vehicle: 1.2.17.23.1. There shall be two pieces tire chocks, one piece hydraulic jack with minimum 5 (five) tons lifting capacity. 1.2.17.23.2. Tool box with padlock (tire iron and rod, two units tire lever, at least 2 (two) kg hammer, 1 (one) wheel wrench, the screwdriver kit consists of min 8 (eight) pieces, the socket wrench kit consists of minimum 200 (two hundred) pieces, 1 (one) grease pump, auto hand lamp). 1.2.17.23.3. Two (2) units rear reflector, signs and warning signs as well as vehicle identification plates, 2 (two) units skid chain, 6 (six) kg fire extinguisher in cargo side, first-aid kit for each vehicle. 1.2.17.24. Wheels/Tyres: The vehicles shall be equipped with the tubeless type radial tires mounted on OEM approved aluminium or steel rims. 1.2.17.24.1. Tires shall be the OEM's recommended tread design for the type of vehicle ordered. .2.17.24.2. As a minimum, all tires shall be all-season tread design and labelled for Mud and Snow (M/S) service. 1.2.17.24.3. All “on ground” tires and wheels shall be identical (i.e. no cross brands or models). 1.2.17.24.4. The vehicles shall be equipped with a full-size spare tire and wheel mounted and secured on a production-type carrier. 1.2.17.24.5. Also all tires shall be produced in last 12 (twelve) months. 1.2.18. Exterior Colour and Logos: The exterior colour shall be selected by the Beneficiary after signature of the contract. 1.2.18.1. Standard cast folio logos shall be applied on proper locations of vehicle body. 1.2.18.2. The requested logo will be put on the vehicles by the Contractor. The final design of the logos will be shared with the Contractor by the Beneficiary. The logos should also be complained with the Visibility Guidelines for European Commission Projects in Turkey published by the EU Delegation to Turkey, at the following address: http://www.avrupa.info.tr/AB_Mali_Destegi/Gorunurluk,Visi.html 1.3. Audio Broadcasting System: There shall be siren featured, with peak lights, external announcement system in driver’s cabin. The audio broadcasting system shall have following specifications. 1.3.1. It shall be compatible with direct current (DC) min. 12V. 1.3.2. It shall have short circuit, high voltage and reverse polarity protection systems. 1.3.3. It shall use vehicle’s battery as an energy source. 1.3.4. It shall have control head which has announcement and speaker selecting feature. The current position of peak lights shall be shown on control head (by means of LED and/or screen). 1.3.5. It shall be compatible with electronic devices (radio, radar etc.) used in the vehicle. 1.3.6. Audio broadcasting system shall be consisting of siren/announcement unit and peak lights unit. 1.3.6.1. Siren and Announcement Unit: 1.3.6.1.1. Siren shall have at least 3 (three) functioning options; manual, slow and fast. 1.3.6.1.2. When slow functioning option is selected, siren sound shall be repeated 12 (twelve) or 13 (thirteen) times in a minute. 1.3.6.1.3. When fast functioning option is selected, siren sound shall be repeated at least 170 (one hundred seventy) times in a minute. 1.3.6.1.4. The output power of siren shall be 100 (one hundred) ±5 (five) Watt (Root Mean Square-RMS) in 27.6 (twenty seven point six) V Direct Current (DC). 1.3.6.1.5. The output power of announcement shall be 50 (fifty) ± 5 (five) Watt (Root Mean Square-RMS) in 27,6 (twenty seven point six) V Direct Current (DC). 1.3.6.1.6. The most frequencies of siren shall be 1640 (sixteen hundred forty) ± 35 (thirty five) Hz and the least frequencies of siren shall be 695 (six hundred ninety five) ± 25 (twenty five) Hz. 1.3.6.1.7. The loudspeaker of siren shall be in the middle of peak lights. 1.3.6.1.8. The loudspeaker of siren shall have the same height of peak lights and black coloured protection. 1.3.6.1.9. The loudspeaker shall have at least N35 grade neodymium magnet. 1.3.6.1.10. The output power of loudspeaker shall be at least 100 Watt, at most 11 (eleven) ohm impedance and 8 (eight) ohm Direct Current (DC) resistance. 1.3.6.1.11. Vibration, durability and high/low tests of siren/announcement unit shall be in compliance with SAE J575 and/or SAE J595 and/or SAE J1849 standards. 1.3.6.2. Peak Lights Unit: 1.3.6.2.1. The height of peak lights unit shall be 11 (eleven) cm (±10%) in highest point, the maximum width shall 40 (forty) cm (±10%) and the length shall be 125 (one hundred twenty five) ± 5 (five) cm. 1.3.6.2.2. Light source of peak light unit shall be Power LED (Light Emitting Diode). Peak Lights Unit shall have white, red and blue LED groups. The brand, model and type of LED’s shall be same in each group. 1.3.6.2.3. Eye protection of all LED’s shall be in compliance with IEC 62471 and/or ANSI/ESNA RP-27 and/or IEC 825-1 and/or EN 60825-1 standards. LED’s shall not contain quicksilver and transmit UV beams. 1.3.6.2.4. LED’s shall have following specifications when they are operating between 2.2 (two point two) and 4,5 (four point five) V Direct Current (DC) with 350 (three hundred fifty) ± 10 (ten) mA; 1.3.6.2.4.1. White LED’S: 1.3.6.2.4.1.1. White LED’s shall have at least 95 (ninety five) lumens power. 1.3.6.2.4.1.2. Colour Rendering İndex (CRI) of white LED’s shall be at least 70 (seventy). 1.3.6.2.4.1.3. Correlated Colour Temperature (CCT) of white LED’s shall be at least between 5500 (fifty five hundred) and 7000 (seven thousand) Kelvin (K). 1.3.6.2.4.2. Red LED’S: 1.3. 6.2.4.2.1. Red LED’s shall have at least 35 (thirty five) lumens power. 1.3.6.2.4.2.2. The wavelength of red LED’s shall be between 620 (six hundred twenty) and 645 (six hundred forty five) nanometer (nm). 1.3.6.2.4.3. Blue LED’S: 1.3.6.2.4.3.1. Blue LED’s shall have at least 20 (twenty) lumens power. 1.3.6.2.4.3.2. The wavelength of blue LED’s shall be between 460 (four hundred sixty) and 490 (four hundred ninety) nanometer (nm). 1.3.6.2.5. Peak lights unit shall have at least 15 (fifteen) red, 15 (fifteen) blue and 6 (six) white LED’s. And these LED’s shall be used in at least 2 (two) groups of 5 (five) most. 1.3.6.2.6. Red LED’s shall be used in the left side (driver’s side), blue LED’s shall be used in right side and white LED’s shall be used in the middle of peak lights unit. 1.3.6.2.7. Peak lights unit shall have at least 2 (two) functioning mods; 1.3.6.2.7.1. In first mode red and blue LED’s shall be flashing in quaternary group 65 (sixty five) ± 10 (ten) times in a minute. 1.3.6.2.7.2. In second mode red and blue LED’s shall be flashing respectively from right to left and from left to right 70 (seventy) ± 5 (five) times in a minute. 1.3.6.2.8. The power (sensitivity) of LED’s shall be at least 2 (two) staged and selectable by users and/or automated to day and night. 1.3.6.2.9. Dust, water, vibration, durability and high/low temperature tests of peak lights unit shall be in compliance with SAE J575 and/or SAE J595 and/or SAE J1849 standards. 1.4. Crime Scene Investigation Vehicle Superstructure: Rear of the vehicle shall be designed of 2 (two) sections (passenger transport and load section). The first section where the personnel will travel and second section where the crime scene investigation equipment will be loaded. 1.4.1. General Requirements: 1.4.1.1. Any assembly operations to be carried out will not cause any weaknesses and failures in the original vehicle's essential security measure or in the aerodynamics of the vehicle. 1.4.1.1. After the assembly process and any design, vehicle and all the parts of superstructure should operate as regular. 1.4.2. Passenger Transport Cabin: 1.4.2.1. Inside of the vehicle shall be divided into 2 (two) sections (passenger transport and load sections) starting from the point where the sliding door is opened. A metal profile with dimensions of 20 (twenty) x 20 (twenty) x 1,5 (one point five) (±10%) mm shall be used for separating the sections. Front and back surfaces of these profiles which will be mounted to the straps shall be covered easily cleaned bright and white PVC material (minimum 3 mm.) over a water-resistant plywood material. The empty space between the plywood and PVC material shall be covered with insulation foams. The profile straps shall be painted with electrostatic dye with the same colour of vehicle inside against rust. 1.4.2.2. Vehicle's floor shall be covered with water-resistant plywood with a minimum thickness of 18 (eighteen) (±10%) mm. The floor of vehicles shall be flattened by filling the spaces with polyurethane foam. Top of the base material shall be covered with plastic based material with a minimum 2 (two) - 2.5 (two point five) (±10%) mm thickness that is bactericidal and easily-cleaned (disinfected), resistant to the chemical materials, nonflammable, anti-static, water/liquid proof. The color of material shall not be very light tones considering that it might be marked, fumed, stained and get dirty. 1.4.2.3. The load and passenger transport sections of the vehicles shall be covered bright white non-flammable insulated PVC material with minimum thickness of 3 (three) (±10%) mm. 1.4.2.4. In the passenger transport section, a single passenger seat shall be mounted to the back of the front passenger seats. The seat, which has minimum 180 (one hundred and eighty) º (degrees) rotation capacity, shall be adjustable with high back head support, dual arm rests and can move back and forth on sledge. The colour of the seat shall be compatible with driver/passenger seats. 1.4.2.5. In the rear side of the passenger transport cabin, an ergonomic and foldable work desk (suitable for laptops) shall be fixed on a suitable place (on wall or cupboard) that may be used in conjunction with the aforesaid seat will be available, 1.4.2.6. There shall be at least 2 (two) lighting system with aluminium body, white-coloured LED with the purpose of lighting in passenger transport cabin. Concealment of the cabling lines shall be ensured. 1.4.2.7. Inside the passenger transport section, a cupboard system shall be installed. The cupboard system shall be designed to meet the following requests at least. The final design of cupboard system will be shared with the Beneficiary by the Contractor. 1.4.2.7.1. This cupboard system shall be designed in conformance with the ergonomic use without affecting the use of the single seat. 1.4.2.7.2. This cupboard system shall have shock absorber type cabinet doors which can be locked. 1.4.2.7.3. Drawer and cupboard systems in the passenger transport section shall be produced from a foam board material with a minimum thickness of 13 (thirteen) (±10%) mm. which is covered with minimum 1 (one) (±10%) mm white PVC. The bottom of the shelves in contact with the materials shall be covered with minimum 3 (three) (±10%) mm thick EVA entirely. 1.4.2.7.4. The drawers shall be lockable with a push button. The necessary safety measures shall be applied in order to prevent damage to the personnel by the drawers in case of an accident. 1.4.2.7.5. The consideration of placing a steel box inside the cupboard will be incorporated into the design. Features of the safe shall be as follows: 1.4.2.7.5.1. Exterior dimensions: 350 (three hundred and fifty) x 250 (two hundred and fifty) x 250 (two hundred and fifty) mm ( ± 50 mm). 1.4.2.7.5.2. Weight: maximum 20 (twenty) kg. 1.4.2.7.5.3. Locking system: password. 1.4.2.7.6. The consideration of placing a vehicle type fridge inside the cupboard will be incorporated into the design. Features of the fridge shall be as follows: 1.4.2.7.6.1. It shall be able to thermostat adjustable cool up to - 18 (minus eighteen) C º. 1.4.2.7.6.2. Net capacity of fridge shall be minimum 10 (ten) lt with maximum size of 65 (sixtyfive) dm3 and max. 15 (fifteen) kg. unload weight. 1.4.2.7.6.3. The product shall be designed to work with 12 (twelve) / 24 (twenty four) V DC (Direct Current) and shall have max. 40 (forty) Watt power. 1.4.2.7.7. A full sine inverter with 0-1 button with a minimum 2000 (two thousand) Watt invertor converting 12 (twelve) DC (Direct Current) V to 220 (two hundred and twenty) ( ± %5) V AC (Alternating Current) fed from 2nd (second) accumulator shall be mounted to an appropriate part of the vehicle. 1.4.2.7.8. There shall be at least 2 (two) 12 (twelve) V car lighter socket, 3 (three) grounding plug with cover transmitting current from a 220 (two hundred and twenty) V AC (Alternating Current) inverter. 1.4.2.7.9. Two hundred and twenty (220) V AC (Alternating Current) shall be available to the plugs. The cables shall be installed inside the sheltered protection lines (cable ducts) mounted on the sidewalls of the vehicle. 1.4.3. Load (CSI Equipment) Transportation Section of the Vehicle: 1.4.3.1. The rear (load transportation part of the vehicle); shall have demountable, portable columns and shelves. The shelves shall have capacity to move upwards and downwards, mountable, adjustable and without any additional process such as welding, etc. And shall be easily mountable on the metal parts of the vehicle. Distance between mounting holes of adjustable shelves shall be max. 5 (five) (±10%) cm. 1.4.3.2. The shelf system with the purpose of stocking crime scene investigation equipment shall be installed on the left and right with minimum 3 (three) shelves each on 2 (two) bearing columns. There shall be fixing apparatus to prevent bags moving. Portable columns of the shelf system will be produced from aluminium in compliance with EN-AW 6063 standards. 1.4.3.3. Metal parts of the shelves shall be stainless anodized coated aluminium. 1.4.3.4. The shelves shall be max. 40 (forty) (±10%) cm. in width and the length of shelves shall be fit longitudinally to the length of load section. 1.4.3.5. There shall be sets on the front and back of the shelves to prevent the bags and materials from falling down. These sets shall be made of the same material as one-piece and the rear part should have a minimum height of 15 (fifteen) (±10%) cm and the front part should have a minimum height of 5 (five) (±10%) cm with special cover system for loading easily. 1.4.3.6. Edges of the shelves shall be coated with stainless material and covered with aluminium. Shelves shall be designed to enable the necessary apparatus mount and dismount easily between them in order to separate them into sections. 1.4.3.7. Inner parts of the shelves shall be covered with high-strength non-skid material. 1.4.3.8. Each one of the shelves shall be able to carry minimum 60 (sixty) (±10%) kg weight. 1.4.3.9. Fixing equipment shall be used in order to fix materials and equipment on the shelves to prevent damage to the personnel and materials during the movement of the vehicle. 1.4.3.10. There shall be at least 2 (two) lighting system with aluminium body, white-coloured LED with the purpose of lighting in load transportation cabin of the vehicle. Concealment of the cabling lines shall be ensured. 1.4.4. Heating and Ventilation System: 1.4.4.1. There shall be self-combustion heating system fed from the fuel tank of vehicles to heat the passenger and driver section. 1.4.4.2. There shall be 2 (two) electric mushroom turbo fans (one is in passenger section and the other is in load section) installed on the roof of vehicles. 1.4.5. Generator: 1.4.5.1. The vehicle shall be equipped with a minimum 2.5 (two point five) KVA power output, wheeled and portable, diesel or gasoline generator. 1.4.5.2. This generator shall be fitted with minimum 220 (two hundred and twenty) V and 12 (twelve) V power outlets which also have overload protection features. 1.4.5.3. Other specifications of the generator: portable, 70 (seventy) dbs. low noise from 7 m, max. 3000 rpm, air cooled, electrical key start, automatic regulation voltage, CE noise compliance, magneto-thermal protection, instrumentation, voltmeter, hour meter, differential circuit breaker, connector for remote start, oil protection with complete set of operator manuals in Turkish and English, fuel gauge, minimum 13 (thirteen) liter fuel tank capacity. 1.4.5.4. Generator shall have 2 (two) mobile extension cords at least 50 (fifty) m long with 4 (four) pieces of grounded outlet. 1.4.6. Roof Mounted Telescopic and Foldable Lighting Tower: 1.4.6.1. Foldable Power Lighting Tower, which can operate telescopically and pneumatically, shall ascend at least 1.8 (one point eight) m above from the vehicle roof and produce at least 14.000 (fourteen thousand) lumens of light by at least 2 (two) light heads, shall be mounted on suitable place of the vehicle roof. 1.4.6.2. It shall have cabled or wireless remote control unit, which has a feature to turn off the system (Park Position) with one touch. Mast shall go down lamps on whichever the position they shall be, shall turn into automatic park position and turn into transport position. 1.4.6.3. It shall have halogen flashing type height warning/caution lamps which can perform 1 (one) x 2 (two) Led/halogen spot lighting. 1.4.6.4. All lighting system (with lamp connection points) shall have the capability to rotate at least 180 (one hundred and eighty) ° (degrees) left and 180 (one hundred and eighty) ° (degrees) right as controlled (up-down in horizontal axis, right-left in vertical axis scanning area). The extreme movements of the lighting system shall be limited on both axes. 1.4.6.5. The folded height of the lighting tower shall be maximum 350 (three hundred and fifty) (±10%) mm. 1.4.6.6. The rotation speed of tower shall be at least 2 (two) round per minute. 1.4.6.7. Working voltage shall be 12 (twelve) or 24 (twenty four) V with DC (Direct Current) and illuminating voltage shall be 220 (two hundred and twenty) V with AC (Alternating Current). 1.4.6.8. There shall be projector protection circuit. Lamp armatures shall be etanj type (water proof) 1.4.6.9. In the case of the vehicle moves when the tower is on (open), there shall be voice alarm in the driver’s cabinet. 1.4.6.10. Total weight of system shall not exceed 65 (sixty five) (±10%) kg. |  |  |  |