# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply of Vehicles for Improving the maritime customs surveillance capacity and operational procedures of TCA - Relaunch

Publication reference: EuropeAid/137274/ID/SUP/TR

Column 1-2 should be completed by the Contracting Authority

Column 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

1.VISIBILITY

All supplies must comply with the generic Visibility policies in force within the scope of external aid contracts financed from the EU general budget; tenderers

will thus be aware that certain visibility rules apply and that the guidelines and manuals concerned may be found in the EUROPEAID website, at:

http://ec.europa.eu/europeaid/funding/communication-and-visibility-manual-eu-external-actions_en

http://avrupa.info.tr/eu-funding-in-turkey/visibility-guidelines.html

Unless otherwise specified, all equipment shall have a solidly fixed and durable label, size 75 mm x 35 mm, as appropriate for each equipment, with the standard EU logo as below:

INSTRUMENT FOR PRE-ACCESSION ASSISTANCE (IPA) PROGRAMME 2007-2013

EuropeAid/137274/ID/SUP/TR

Lot No:

Item No:

Serial No:

%85 Avrupa Birliği Katkısı, %.15 Ulusal Katkı ile alınmıştır

EU Contribution  85% National Contribution 15%

GENERAL REQUIREMENTS

When required by national and/or international regulations, all the costs relating to any permission, licencing, certification, testing, calibration or other documentation of the supplies shall be borne by the Contractor.

Trainings will be provided by the Contractor for two (2) officers for each vehicle at the delivery places.

For pick-up vehicles; one-day training by a certified automotive engineer/technician for two (2) officers at the place of delivery including the use of the vehicles, understanding of their qualities, vehicle options and capabilities of a 4WD vehicle, differences in driving a 4WD on-road and off-road, passenger and vehicle safety precautions, fixing simple problems (such as  tire blowout and changing the motor oil, etc.), the use of LED flashing lights bar, siren announcement and light control device, vehicle radio system, and LED programmable message sign. All costs related to training inclusive of training materials (such as brochures of the vehicle and equipment) will be met by the Contractor.

For small vehicles; one-day training by a certified automotive engineer/technician for two (2) officers at the place of delivery including the use of the vehicles, understanding of their qualities, fixing simple problems (such as tire blowout and changing the motor oil, etc.), the use of LED flashing lights bar, siren announcement and light control device, vehicle radio system, and LED programmable message sign. All costs related to training inclusive of training materials (such as brochures of the vehicle and equipment) will be met by the Contractor.

All the technical features indicated throughout Technical Specifications document must be referred on the technical documents/brochures/catalogues submitted by the Tenderers in their Technical Offer.

Special designed vehicles

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1.1 | Pick-Up Vehicles |  |  |  |
| 1.1.1 | General Technical Features |  |  |  |
| 1.1.1.1 | Must have a diesel engine with at least 120 HP max power and at least 320 Nm max torque. |  |  |  |
| 1.1.1.2 | Must have an automatic or manual transmission. |  |  |  |
| 1.1.1.3 | Must have minimum 60 litre fuel tank. The Contractor must provide full tank of fuel, while delivering the vehicle. |  |  |  |
| 1.1.1.4 | Must comply with Euro emission standards pursuant to the current Turkish legislative regulations |  |  |  |
| 1.1.1.5 | Must consume less than 10.0 litres / 100 km for city/highway combination fuel consumption (according to catalogue values). |  |  |  |
| 1.1.1.6 | Must be 4-wheel drive. |  |  |  |
| 1.1.1.7 | Must have a Double-Cab with a seat capacity for at least 4 people. |  |  |  |
| 1.1.1.8 | Must have 4 doors. Picture must be demonstrated on the catalogue both for interior and exterior design. |  |  |  |
| 1.1.2 | Size |  |  |  |
| 1.1.2.1 | External width (excluding side mirrors) must be minimum 1700 mm. |  |  |  |
| 1.1.2.2 | External length must be minimum 5000 mm. |  |  |  |
| 1.1.2.3 | External height must be minimum 1700 mm. |  |  |  |
| 1.1.3 | Safety and Security |  |  |  |
| 1.1.3.1 | Must have front disc brakes and rear drum brakes. |  |  |  |
| 1.1.3.2 | Must have remote control central locking system. |  |  |  |
| 1.1.3.3 | Must have ABS (Antilock Brake System). |  |  |  |
| 1.1.3.4 | Must have ESP system or equivalent. |  |  |  |
| 1.1.3.5 | Must have driver and passenger air bags. |  |  |  |
| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| 1.1.3.5 | Must have a park sensor system with adequate number of sensors inside the back bumper. Sensor system must give the distance information with audible alarms. There must be also a separate display showing the distance. |  |  |  |
| 1.1.3.6 | The vehicle shall be equipped with a fire extinguisher mounted in an appropriate place, in accordance with the legal regulations. |  |  |  |
| 1.1.4 | Interior |  |  |  |
| 1.1.4.1 | Must have a hydraulic steering wheel with adjustable height options. |  |  |  |
| 1.1.4.2 | Must have an air conditioner. |  |  |  |
| 1.1.5 | Exterior |  |  |  |
| 1.1.5.1 | Must have electrical side mirrors. |  |  |  |
| 1.1.5.2 | Must have front fog lights incorporated into the front bumper. |  |  |  |
| 1.1.5.3 | Load area must be covered with a durable and folding cover with locking system. |  |  |  |

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1.1.6 | Law Enforcement Design |  |  |  |
| 1.1.6.1 | The colour of the vehicle must be white. If the Contractor proves by an official document endorsed by the main distributer of the vehicle in Turkey that providing the vehicles having white colour is not possible at that moment due to the reasons which are specified in that official document, light grey colour could be accepted by the Beneficiary. |  |  |  |
| 1.1.6.2 | On the exterior part of the vehicle, logos and/or inscriptions of Turkish Customs Administration must be available. There shall also be reflective coloured stickers on the vehicle. The dual colour LED flashing lights which has waterproof protection and inverse current protection must be located inside the front grill of the vehicle. The light bar must be mounted onto the top of the vehicle in an appropriate place. The technical features of the light bar: - Dual colour (red and blue) - 9 modules comprised of high power LED light sources - Illuminated in 5 different modes - Front and back parts of the LED modules must be operated and illuminated separately. - Optic lenses must be made up of lexan polycarbonate which is not effected by external conditions. A water test shall be implemented on the top of the vehicle in order to check if the insulation is properly made between the connections of the light bar and vehicle in order to prevent any liquid leakage into the vehicle. A siren announcement and light control device must be provided, which will be operated with the light bar on the top of the vehicle and LED flashing lights inside the front grill. The device must be protected against short circuit and inverse current connection. A speaker must be mounted inside the bumper or on the top of the vehicle. The details of the location, size, design and quality of the stickers, labels and insignia will be provided to the Contractor. |  |  |  |
| 1.1.6.3 | A fixed vehicle radio system must be installed inside the vehicle: - Frequency band must be between 136 and 174 MHz. - Number of channels must be minimum 12. - RF output must be indicated in Watts. - Frequency control must be made via synthesizer. - The system must have a hand microphone, a connection place to hang the microphone and a power cable. - Any equipment and cable that must be used to install and assembly the system shall be provided by the Contractor. Installation and assembly shall be carried out by the Contractor. - The VHF/FM vehicle radio system must conform to the relevant legal regulations issued by the “Information and Communication Technologies Authority” of Turkey. - The system must be provided with an antenna. The connection between the system and the antenna must be made by the Contractor. |  |  |  |

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1.1.6.4 | A LED programmable message sign must be mounted on the rear window of the vehicle. Minimum technical specifications of the sign is below: - Must store at least 12 messages which can be pre-recorded - Messages must be controlled and changed via a control unit easily accessible by the driver/passenger. - Must be fed from vehicle battery directly. - Display dimension must be at least 40 cm x 10 cm. - Messages must be viewed in bright red colour and seen from at least 20 meters distance. - Characters on the message sign must be compatible with Turkish characters. |  |  |  |
| 1.1.6.5 | If it is required to get a licence or certificate from Turkish Standards Institute (TSE) for the interior and exterior design of the vehicle, it must be undertaken by the Contractor. The certificate must conform with and accompany the technical documents of the vehicle. |  |  |  |
| 1.1.7 | Other |  |  |  |
| 1.1.7.1 | Must have a set of tools that provided within the vehicle including at least double ended ring spanner set, double ended open jaw spanner set, socket set, hammers, screwdriver set, hexagon key sets, adjustable wrenches, plier set, electrical drill, scissors, tire chain set (for driving on snow and ice), air gauge and pump, tire jack and lever, mobile lamp (with a cable of minimum 5 meters), reflector, chain cutter, wedge and tow. A large toolbox and a transportation bag with reflective tape clearly marked “Customs Enforcement” (Turkish and English) logo details of which will be determined by the Beneficiary must be provided for the set of tools. |  |  |  |
| 1.1.7.2 | Traffic set including first aid kit in accordance with the legal regulations placed in a tough transportation bag with reflective tape on which “Customs Enforcement” (Turkish and English) logo details of which will be determined by the Beneficiary is clearly marked must be provided. |  |  |  |
| 1.1.7.3 | 5 reflective material vests and 5 raincoats on the back parts of which are clearly marked “Customs Enforcement” (Turkish and English) logo details of which will be determined by the Beneficiary and 50 pairs of protective gloves with slip-resistant coating for the vehicle rummage. The colour and design of the vests shall be determined by the Beneficiary. |  |  |  |
| 1.1.7.4 | 10 traffic cones which can be folded into a flat shape and has a reflective surface and a light apparatus that can be mounted onto the traffic cone must be provided. |  |  |  |
| 1.1.7.5 | Two tubes of foam must be provided in order to repair the flat tire temporarily. |  |  |  |
| 1.1.7.6 | A metal battering ram must be provided to break the locked doors. |  |  |  |
| 1.1.7.7 | Reflective bands on which “Gümrük Muhafaza – Girilmez” in Turkish details of which will be determined by the Beneficiary sign is indicated must be provided. These bands must have a length of minimum 5000 meters. The bands shall be cut easily by scissors. |  |  |  |
| 1.1.7.8 | Four spare snow tires and one regular spare tire, all of which are the same brand with the tires installed on the vehicle, must be provided. |  |  |  |
| 1.1.7.9 | The van must be delivered to the Beneficiary after all the administrative and legal procedures concerning the licence plate, traffic insurance and technical examination by certified company are finalized. If it is required for Beneficiary to complete all or some of these procedures, the cost must be born by the Contractor. |  |  |  |

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1.2 | Small Vehicles |  |  |  |
| 1.2.1 | General Technical Features |  |  |  |
| 1.2.1.1 | Must have a diesel engine with at least 100 HP max power and at least 220 Nm max torque. |  |  |  |
| 1.2.1.2 | Must have a manual or automatic gearbox. In case a manual gearbox is offered, it should have minimum 5 speed. |  |  |  |
| 1.2.1.3 | Must have minimum 60 litre fuel tank. The Contractor must provide full tank of fuel, while delivering the vehicle. |  |  |  |
| 1.2.1.4 | Must comply with Euro emission standards pursuant to the current Turkish legislative regulations. |  |  |  |
| 1.2.1.5 | Must consume less than 6.0 litres / 100 km for city/highway combination fuel consumption (according to catalogue values) |  |  |  |
| 1.2.2 | Size and Capacity |  |  |  |
| 1.2.2.1 | External width (excluding side mirrors) must be minimum 1780 mm. |  |  |  |
| 1.2.2.2 | External length must be minimum 4270 mm. |  |  |  |
| 1.2.2.3 | External height must be minimum 1800 mm. |  |  |  |
| 1.2.3 | Safety and Security |  |  |  |
| 1.2.3.1 | Must have front disc and rear disc/drum brakes. |  |  |  |
| 1.2.3.2 | Must have remote control central locking system. |  |  |  |
| 1.2.3.3 | Must have ABS (Antilock Brake System) |  |  |  |
| 1.2.3.4 | Must have an ESP system or equivalent. |  |  |  |
| 1.2.3.5 | Must have driver, passenger and side air bags. |  |  |  |
| 1.2.3.6 | Must have a park sensor system with adequate number of sensors inside the back bumper. Sensor system must give the distance information with audible alarms. There must be also a separate display showing the distance. |  |  |  |
| 1.2.3.7 | The vehicle shall be equipped with a fire extinguisher mounted in an appropriate place, in accordance with the legal regulations. |  |  |  |

| 1.2.4 | Interior |  |  |  |
| --- | --- | --- | --- | --- |
| 1.2.4.1 | Must have a hydraulic steering wheel with adjustable height and depth options. |  |  |  |
| 1.2.4.2 | Must have an air conditioner. |  |  |  |
| 1.2.5 | Exterior |  |  |  |
| 1.2.5.1 | Must have electrical side mirrors. |  |  |  |
| 1.2.5.2 | On the both sides of the rear compartment, there must be sliding doors. |  |  |  |
| 1.2.5.3 | Must have front fog lights incorporated into the front bumper. |  |  |  |
| 1.2.5.4 | Van must have three double windows on both side of the vehicle. One for driver part, one for passenger part and one for the boot. Passenger part and boot windows, and also boot door window must be tinted in accordance with the legal regulations. |  |  |  |
| 1.2.6 | Law Enforcement Design |  |  |  |
| 1.2.6.1 | The colour of the vehicle must be white. If the Contractor proves by an official document endorsed by the main distributer of the vehicle in Turkey that providing the vehicles having white colour is not possible at that moment due to the reasons which are specified in that official document, light grey colour could be accepted by the Beneficiary. |  |  |  |
| 1.2.6.2 | On the exterior part of the vehicle, logos and/or inscriptions of Turkish Customs Administration must be available. There shall also be reflective coloured stickers on the vehicle. The dual colour LED flashing lights which has waterproof protection and inverse current protection must be located inside the front grill of the vehicle. The light bar must be mounted onto the top of the vehicle in an appropriate place. The technical features of the light bar: - Dual colour (red and blue) - 9 modules comprised of high power LED light sources - Illuminated in 5 different modes - Front and back parts of the LED modules must be operated and illuminated separately. - Optic lenses must be made up of lexan polycarbonate which is not effected by external conditions. A water test shall be implemented on the top of the vehicle in order to check if the insulation is properly made between the connections of the light bar and vehicle in order to prevent any liquid leakage into the vehicle. A siren announcement and light control device must be provided, which will be operated with the light bar on the top of the vehicle and LED flashing lights inside the front grill. The device must be protected against short circuit and inverse current connection. A speaker must be mounted inside the bumper or on the top of the vehicle. The details of the location, size, design and quality of the stickers, labels and insignia will be provided to the Contractor. |  |  |  |
| 1.2.6.3 | A fixed vehicle radio system must be installed inside the vehicle: - Frequency band must be between 136 and 174 MHz. - Number of channels must be minimum 12. - RF output must be indicated in Watts. - Frequency control must be made via synthesizer. - The system must have a hand microphone, a connection place to hang the microphone and a power cable. - Any equipment and cable that must be used to install and assembly the system shall be provided by the Contractor. Installation and assembly shall be carried out by the Contractor. - The VHF/FM vehicle radio system must conform to the relevant legal regulations issued by the “Information and Communication Technologies Authority” of Turkey. - The system must be provided with an antenna. The connection between the system and the antenna must be made by the Contractor. |  |  |  |
| 1.2.6.4 | A LED programmable message sign must be mounted on the rear window of the vehicle. Minimum technical specifications of the sign is below: - Must store at least 12 messages which can be pre-recorded. - Messages must be controlled and changed via a control unit easily accessible by the driver/passenger. - Must be fed from vehicle battery directly. - Display dimension must be at least 40 cm x 10 cm. - Messages must be viewed in bright red colour and seen from at least 20 meters distance. Characters on the message sign must be compatible with Turkish characters. |  |  |  |
| 1.2.6.5 | If it is required to get a licence or certificate from Turkish Standards Institute (TSE) for the interior and exterior design of the vehicle, it must be undertaken by the Contractor. The certificate must conform with and accompany the technical documents of the vehicle. |  |  |  |

| 1.2.7 | Other |  |  |  |
| --- | --- | --- | --- | --- |
| 1.2.7.1 | Must have a set of tools that provided within the vehicle including at least double ended ring spanner set, double ended open jaw spanner set, socket set, hammers, screwdriver set, hexagon key sets, adjustable wrenches, plier set, electrical drill, scissors, tire chain set (for driving on snow and ice), air gauge and pump, tire jack and lever, mobile lamp (with a cable of minimum 5 meters), reflector, chain cutter, wedge and tow. A transportation bag with reflective tape clearly marked “Customs Enforcement” (Turkish and English) logo details of which will be determined by the Beneficiary must be provided for the set of tools. |  |  |  |
| 1.2.7.2 | Traffic set including first aid kit in accordance with the legal regulations placed in a tough transportation bag with reflective tape on which “Customs Enforcement” (Turkish and English) logo details of which will be determined by the Beneficiary is clearly marked must be provided. |  |  |  |
| 1.2.7.3 | 5 reflective material vests and 5 raincoats on the back parts of which are clearly marked “Customs Enforcement” (Turkish and English) logo details of which will be determined by the Beneficiary and 50 pairs of protective gloves with slip-resistant coating for the vehicle rummage. The colour and design of the vests shall be determined by the Beneficiary. |  |  |  |
| 1.2.7.4 | 10 traffic cones which can be folded into a flat shape and has a reflective surface and a light apparatus that can be mounted onto the traffic cone must be provided. |  |  |  |
| 1.2.7.5 | Two tubes of foam must be provided in order to repair the flat tire temporarily. |  |  |  |
| 1.2.7.6 | A metal battering ram must be provided to break the locked doors. |  |  |  |
| 1.2.7.7 | Reflective bands on which “Gümrük Muhafaza – Girilmez” in Turkish details of which will be determined by the Beneficiary sign is indicated must be provided. These bands must have a length of minimum 5000 meters. The bands shall be cut easily by scissors. |  |  |  |
| 1.2.7.8 | Four spare snow tires and one regular spare tire, all of which are the same brand with the tires installed on the vehicle, must be provided. |  |  |  |
| 1.2.7.9 | The van must be delivered to the Beneficiary after all the administrative and legal procedures concerning the licence plate, traffic insurance and technical examination by certified company are finalized. |  |  |  |

APPENDIX to Annex II

Indicative delivery places of vehicles are as follows:

| 5 Small Vehicles | Quantity | 4 Pick-up Vehicles | Quantity |
| --- | --- | --- | --- |
| İskenderun Anti-Smuggling and Intelligence District Directorate of Customs Enforcement | 1 | İskenderun Anti-Smuggling and Intelligence District Directorate of Customs Enforcement | 1 |
| Mersin Anti-Smuggling and Intelligence District Directorate of Customs Enforcement | 1 | Mersin Anti-Smuggling and Intelligence District Directorate of Customs Enforcement | 1 |
| Mersin Anti-Smuggling and Intelligence District Directorate of Customs Enforcement (Taşucu Section) | 1 | Mersin Anti-Smuggling and Intelligence District Directorate of Customs Enforcement (Taşucu Section) | 1 |
| Antalya Anti-Smuggling and Intelligence District Directorate of Customs Enforcement | 1 | Antalya Anti-Smuggling and Intelligence District Directorate of Customs Enforcement | 1 |
| Marmaris Anti-Smuggling and Intelligence District Directorate of Customs Enforcement | 1 |  |  |